Below is a beginner-friendly, step-by-step framework that farm-machine companies and agricultural planners commonly use to decide where knapsack sprayers remain rational, where automotive (mobile) sprayers should be introduced, and how many mobile sprayers are needed.
I will keep assumptions explicit and show real calculations with numbers, so you can adapt them to your own area.

1. Problem restated in simple terms

You want to answer three practical questions:

- Which fields should continue using knapsack sprayers?

- Which fields should switch to automotive sprayers?

- How many automotive sprayers are required in total for your area?

To answer this, we compare cost, efficiency, and feasibility for each field type.

2. Key factors that affect sprayer choice (beginner overview)

2.1 Land factors

| Factor | Why it matters |
| --- | --- |
| Field size (ha) | Larger fields favor machines |
| Terrain (flat vs mountainous) | Machines need access & stability |
| Shape / accessibility | Irregular or terraced land favors knapsack |
| Road access | Mobile sprayers require access paths |

2.2 Crop factors

| Crop type | Spraying characteristics |
| --- | --- |
| Rice, wheat, maize | Large, uniform → machine-friendly |
| Vegetables | Smaller plots, frequent spraying |
| Orchards / terraces | Slopes → knapsack often better |
| High-value crops | Labor cost is more critical |

2.3 Economic factors

| Item | Knapsack sprayer | Automotive sprayer |
| --- | --- | --- |
| Purchase price | Very low | High |
| Labor required | Very high | Low |
| Operating cost/ha | High | Low |
| Daily coverage | Very small | Very large |

3. Define basic technical & economic assumptions (example)

You must define these first, otherwise no calculation is meaningful.

3.1 Sprayer performance assumptions

| Parameter | Knapsack | Automotive |
| --- | --- | --- |
| Effective spraying capacity | 0.5 ha/day/person | 10 ha/day/machine |
| Workers required | 1 per sprayer | 1 operator |
| Fuel / energy cost | negligible | $5 / ha |

3.2 Cost assumptions (example values)

| Item | Value |
| --- | --- |
| Labor cost | $20 / day |
| Knapsack sprayer price | $150 |
| Automotive sprayer price | $15,000 |
| Spraying frequency | 5 times / season |
| Working days per season | 60 days |

4. Field classification method (VERY IMPORTANT)

You do not evaluate all land together.
You classify fields first.

4.1 Example field categories

| Category | Field size | Terrain | Typical crops |
| --- | --- | --- | --- |
| A | < 1 ha | Mountainous / terraces | Vegetables, orchards |
| B | 1–3 ha | Mixed | Vegetables, maize |
| C | > 3 ha | Flat / lowland | Rice, wheat |

5. Cost comparison per hectare (core logic)

5.1 Knapsack sprayer cost per hectare

Labor time per hectare

Time=10.5=2 days/ha

Labor cost

2×$20=$40/ha

Total per spray

$40/ha

Seasonal cost (5 sprays)

5×40=$200/ha/season

5.2 Automotive sprayer cost per hectare

Labor time

110=0.1 day/ha

Labor cost

0.1×$20=$2/ha

Fuel cost

$5/ha

Total per spray

$7/ha

Seasonal cost

5×7=$35/ha/season

6. Decision rule (simple & practical)

| Condition | Recommended sprayer |
| --- | --- |
| Field < 1 ha OR mountainous OR no road | Knapsack |
| Field 1–3 ha | Depends on labor availability |
| Field > 3 ha, flat, road access | Automotive |

7. Example area-wide calculation

7.1 Example farmland distribution

| Category | Number of fields | Avg size | Total area |
| --- | --- | --- | --- |
| A | 300 | 0.5 ha | 150 ha |
| B | 120 | 2 ha | 240 ha |
| C | 80 | 5 ha | 400 ha |
| Total | 500 | — | 790 ha |

7.2 Assign sprayer type

| Category | Sprayer |
| --- | --- |
| A | Knapsack |
| B | Mixed (50% mobile) |
| C | Automotive |

7.3 Area served by automotive sprayers

- Category B:

240×50%=120 ha

- Category C:

400 ha

Total mobile-sprayer area

120+400=520 ha

8. Calculate number of automotive sprayers needed

8.1 Seasonal capacity per sprayer

10 ha/day×60 days=600 ha/season

8.2 Required sprayers

520600=0.87

Apply practical buffer (weather, breakdowns, peak demand):

0.87×1.3≈1.13

➡ Conclusion: 2 automotive sprayers required

9. How this supports product-development decisions

| Question | Answer this model gives |
| --- | --- |
| Market size | Total sprayers needed |
| Adoption rate | % of land suitable |
| ROI for farmers | Cost/ha reduction |
| Target customers | Large lowland farms |
| Product variants | Flat vs hilly models |

10. Offline tool you can use (no programming required)

Recommended tool: Microsoft Excel / LibreOffice Calc

How to structure it

Sheet 1 – Input

- Field ID

- Area (ha)

- Terrain type

- Crop

- Road access (Yes/No)

Sheet 2 – Parameters

- Labor cost

- Sprayer prices

- Capacity values

Sheet 3 – Decision logic

=IF(AND(A2>3, C2="Flat", D2="Yes"), "Automotive", "Knapsack")

Sheet 4 – Summary

- Total area by sprayer type

- Required sprayers

- Cost savings

This is exactly how manufacturers and extension agencies do early feasibility studies.

11. Recommended literature search keywords (English)

Use these in Google Scholar / Scopus / Web of Science:

- Agricultural sprayer mechanization economics

- Farm machinery adoption decision model

- Cost–benefit analysis of pesticide application methods

- Labor productivity in agricultural spraying

- Smallholder mechanization thresholds

- Field size effect on farm machinery adoption

- Terrain impact on agricultural mechanization

- Custom hiring services agricultural machinery

- Technology diffusion in agricultural machinery

- Precision spraying vs manual spraying economics

12. Key beginner takeaway

- Do not replace knapsack sprayers everywhere

- Introduce mobile sprayers only where they reduce total cost

- Field size + terrain + labor cost drive everything

- This method gives you a quantitative, defendable justification for new product development

If you want, I can:

- Build a ready-to-use Excel template

- Customize numbers for your country or crop

- Extend this into a 5-year market forecast